EGR-338

Microcontrollers in Smart Systems

LABORATORY 2.19

Week 3

Wednesday January 24, 2024

Due Wednesday January 31, 2024

Names:

Saif Elsaady

Scott Lidman

Elton Salt

Marla Hawthorne

Kevis Seymore

COMPARATORS

Goals

The goal of this lab is to familiarize you with the Comparator Module C1 on the PIC16F887 Microcontroller.

In this lab you will establish a CVREF Voltage and compare it to another  Comparator Input C12IN1 - Voltage, resulting in either a high or low output state at Comparator Output C1OUT to turn and LED ON or OFF.

Following, a Sinewave will be applied to the Comparator input C12IN1 – and the results displayed on the Digilent Analog Discovery Board from the Comparator Output C1OUT.

Lastly, CVREF Voltage will be Modified in C-Code and Demonstrated.

Note: from time-to -time check the DC Voltage out of your LM7805 Voltage Regulator for a solid 5 V.  The 9 V Batteries lose charge with time.  When the Voltage output from the LM7805 Regulator drops below a threshold the Microcontroller no longer operates correctly and it will appear that your C-Code will not Compile.

Procedure:

- Compile Provided Comparator Module C1 C-Code and Demonstrate using a CdS Cell Photo Detector PDV-P8103 and Resistor Voltage Divider for faster response.

Measured Voltage at which Comparator Module C1 transitions the LED RA4 Pin 6

- Using the Digilent Analog Discovery Board, apply a Sinewave from the WaveGen. Begin with 1kHz, 4Vpk and 500us/div time base to RA1 Pin 3 Input Voltage to be Compared against CVREF.  Then observe using the Scope, the resulting waveform from RA4 Pin 6.

Demonstrate for sign off by Instructor Both Waveforms in time on the Digilent Analog Discovery Board Scope

- Modify the Provided C-Code and Change CVREF Voltage and Verify and Demonstrate to Instructor for Sign off (Provide you Calculations and Expected CVREF Voltage below)

Calculations

Expected CVREF Voltage

Measured Voltage at which Comparator Module C1 transitions the LED RA4 Pin 6

Instructor Sign Off:

- Compile Provided Comparator Module C1 C-Code and Demonstrate using a CdS Cell Photo Detector PDV-P8103 and Resistor Voltage Divider for faster response

0XAF = 175 → 175/255 = 0.686 * 5Vmax = 3.4V

Measured Voltage at which Comparator Module C1 transitions the LED RA4 Pin 6 ~3V

- Using the Digilent Analog Discovery Board, apply a Sinewave from the WaveGen. Begin with 1kHz, 4Vpk and 500us/div time base to RA1 Pin 3 Input Voltage to be Compared against CVREF.  Then observe using the Scope, the resulting waveform from RA4 Pin 6.

Demonstrate for sign off by Instructor Both Waveforms in time on the Digilent Analog Discovery Board Scope __________________

Enter Pictures

- Modify the Provided C-Code and Change CVREF Voltage and Verify and Demonstrate to Instructor for Sign off (Provide you Calculations and Expected CVREF Voltage below) __________________

Calculations 0X64 = 100 → 100/255 = 0.392 * 5Vmax = 1.96V

Expected CVREF Voltage 1.96V

Measured Voltage at which Comparator Module C1 transitions the LED RA4 Pin 6 ~2V